Lin-dare Automotive

Address: 113 Spring St, Ossining, NY 10562	Phone: (914) 923-5525	Website: lindareauto.com

POST-ACCIDENT VEHICLE INSPECTION REPORT

Report Date: April 12, 2026
Inspector Name: Mike Chen, Certified Auto Inspector
Vehicle Involved in Accident on: April 10, 2026

1. VEHICLE INFORMATION

- Make/Model: Honda Civic LX

- Year: 2022

- License Plate: 6XYZ543

- VIN (last 4 digits): 8921

- Odometer: 31,450 miles

2. DAMAGE SUMMARY (Check all that apply)

- Front Bumper / Grille

- Hood

- Left Front Fender

- Right Front Fender

- Left Rear Quarter Panel

- Right Rear Quarter Panel

- Rear Bumper

- Roof

- Windshield (cracked)

- Driver Door

- Passenger Door

- Lights (head/tail/signal) – left headlight only

- Airbags deployed? Yes / No

3. DETAILED DAMAGE DESCRIPTION

| Area | Type of Damage | Severity (Mild/Mod/Severe) |
| --- | --- | --- |
| Front bumper | Shattered, hanging on right side | Severe |
| Hood | Bent upward at center, unable to latch | Moderate |
| Left front fender | Deep scratches, pushed back into door gap | Severe |
| Left headlight | Housing cracked, bulb broken | Moderate |
| Radiator support | Slight bowing visible | Moderate |

4. FUNCTIONAL CHECKS (After accident)

- Engine starts: No – cranks but won't start (fuel cut-off active?)

- Steering: Stiff – power steering fluid low / possible pump damage

- Brakes: Functional but soft pedal

- Fluid leaks: Yes – green coolant dripping from front right

- Warning lights on dash: Yes – airbag, check engine, brake system

5. SAFETY NOTES

- Seatbelts locked / damaged – driver seatbelt retractor stuck

- Airbags deployed – driver and passenger front airbags

- Structural/frame damage suspected: Yes – front frame rail on driver side slightly crumpled

6. RECOMMENDATION

- Repairable

- Potentially totaled (requires shop estimate)

- Not safe to drive – tow required

7. ADDITIONAL COMMENTS
Vehicle was towed to inspection location. Coolant leak significant. No fluids reached windshield or cabin. Driver reported no injuries. Recommend insurance adjuster review for total loss determination due to frame and airbag deployment.

Inspector Signature: Mike Chen